Prompt (Pro version):

make a 75 minute lecture with active learning strategies.

Output:

Here is a comprehensive 75-minute lecture plan for MEEG 3379 Elements of Mechanical Design, incorporating active learning strategies to keep students engaged with the material.

### Course & Instructor Introduction (5 Minutes)

- Welcome & Introductions: Welcome the class to MEEG 3379 Elements of Mechanical Design. Introduce yourself as Dr. Shweta Dabetwar.

- Support & Logistics: Introduce the Teaching Assistant, Colby Martinez (cjmartinez@ualr.edu). Remind students of your office hours: Every Thursday from 12:15 to 1:15 pm in EIT 544 (sdabetwar@ualr.edu).

- Course Material: Introduce the reference textbook: Shigley's Mechanical Engineering Design, 11th Edition, by Richard Budynas (McGraw Hill, ISBN: 978-981-315-898-6).

### Segment 1: The Concept of Failure (15 Minutes)

Objective: Define failure and understand its real-world implications.

- Topic Introduction: Introduce Chapter 5, which focuses on failures resulting from static loading.

- Defining Failure: Ask the foundational question: What is failure?. Explain that failure occurs when a component or a system of components cannot do its intended job.

- Modes of Failure: Detail that failure includes a part separating into pieces, permanent distortion, downgraded reliability, or compromised function.

- The Root Cause: Explain that systems or components ultimately fail due to diminished strength.

Active Learning: Think-Pair-Share (5 mins)

- Prompt: Ask students to think of a time a mechanical device they owned "failed" without explicitly breaking into pieces. * Action: Have them pair up for 2 minutes to share their examples, focusing on categorizing their examples into the failure modes discussed (e.g., permanent distortion vs. compromised function). Ask 2-3 pairs to share with the room.

### Segment 2: The "Why" of Failure & Design Categories (25 Minutes)

Objective: Understand the statistical nature of material strength and how manufacturing scales dictate testing.

- The Testing Paradox: Pose the question: If all manufacturing parts go through extensive testing, why does failure still happen?.

- Statistical Strength: Explain that the strengths of all mass-produced parts will be different due to variations in dimensions, machining, forming, and composition. Emphasize that strength is statistical in nature, characterized by a mean, standard deviations, and distributional identification. Therefore, engineers cannot simply rely on published values of yield or ultimate strength.

- Operational Realities: Stress that testing is critical to duplicate the actual design situation. Note that stress concentration during operation can throw everything off.

- Four Design Categories: Introduce the four categories of design based on testing feasibility:

  - The part's failure endangers human life or is made in extremely large quantities, justifying an elaborate testing program.

  - The part is made in large enough quantities to make a moderate series of tests feasible.

  - The part is made in small quantities where testing isn't justified, or the design timeline is too rapid for testing.

  - The part is already designed, manufactured, tested, and found unsatisfactory, requiring analysis to understand why and how to improve it.

Active Learning: Small Group Case Studies (10 mins)

- Action: Divide the class into groups of 3-4. Display the two discussion questions on the board. * Prompt 1: If you are hired at a startup with limited resources and fewer testing facilities, but your product needs very high accuracy, what measures would you take?. * Prompt 2: If you are a design engineer working in each of the four design categories, what specific measures would you take?. * Debrief: Have groups report their strategies for the startup scenario.

### Segment 3: Understanding Static Loads (15 Minutes)

Objective: Define static loading and identify its various forms in mechanical design.

- Definition: Define a static load as a stationary force or couple applied to a member.

- Conditions for a Static Load: Explain that to remain stationary, the force or couple must not change in magnitude, direction, or point(s) of application.

- Types of Static Loads: Outline that a static load can produce axial tension or compression, shear load, bending load, torsional load, or a combination of two or more of these loads.

Active Learning: Visual Identification (5 mins)

- Action: Show pictures of various everyday structures or mechanical parts (e.g., a bridge cable, a driveshaft, a diving board). * Prompt: Call on students to identify which type of static load is primarily acting on the member (axial, shear, bending, or torsional).

### Segment 4: Failure Theories Background (15 Minutes)

Objective: Distinguish between ductile and brittle materials and introduce the accepted failure theories.

- Ductile vs. Brittle: Explain the background of failure theories.

  - If the failure strain is $\varepsilon_f \geq 0.05$, there is an identifiable yield strength, indicating the material is ductile. In this case, the maximum stress is the yield stress, and generally, tensile and compressive values are similar.

  - If the failure strain is $\varepsilon_f < 0.05$, there is no identifiable yield strength, indicating the material is brittle. In this case, the maximum stress is ultimate stress, and tensile and compressive values are generally different.

- The Reality of Theories: Clarify that there is no universal theory for failure. Over the years, several hypotheses have been formulated and tested, leading to today's accepted practices, which we refer to as "theories".

- Mapping the Theories: Briefly outline the generally accepted failure theories to set up the next lecture:

  - Ductile Materials (Yield Criteria): Maximum shear stress (MSS), Distortion energy (DE), and Ductile Coulomb-Mohr (DCM).

  - Brittle Materials (Fracture Criteria): Maximum normal stress (MNS), Brittle Coulomb-Mohr (BCM), and Modified Mohr (MM).

Active Learning: "Muddiest Point" Exit Ticket (5 mins) * Action: In the final minutes, ask students to write down the clearest concept they learned today and the "muddiest" (most confusing) point regarding the transition from material strain to failure theories.

- Closing: Collect these to review before the next class. Remind them of Thursday office hours.

Prompt (Fast version):

make a 75 minute lecture with active learning strategies.

This 75-minute lecture plan is designed for MEEG 3379: Elements of Mechanical Design. It balances direct instruction on static loading failure with active learning strategies to help students navigate the statistical nature of engineering design.

## Lecture Overview

- Course: MEEG 3379 Elements of Mechanical Design

- Instructor: Dr. Shweta Dabetwar

- Topic: Chapter 5: Failures Resulting from Static Loading

- Textbook: Shigley’s Mechanical Engineering Design, 11th Ed.

## I. Introduction & Logistics (5 Minutes)

- Admin: Introduce Dr. Dabetwar (Office: EIT 544) and TA Colby Martinez.

- Textbook Overview: Ensure everyone has access to the 11th edition of Shigley’s.

- Goal: Move beyond "breaking" things to understanding the engineering criteria for failure.

## II. Defining Failure & The Statistical Reality (15 Minutes)

- The Definition: Failure isn't just a part snapping ; it includes permanent distortion, loss of reliability, or a system being unable to perform its intended job.

- The "Why": Failure happens because of diminished strength.

- The Statistical Nature: Even mass-produced parts vary in strength due to dimensions, machining, and composition. We cannot simply use "book values" for yield strength because strength has a mean and standard deviation.

### Active Learning: Think-Pair-Share

Prompt: If we can't use published yield strength values directly because of statistical variation, how do you think we should account for this in a real design?

- 1 minute: Individual thinking.

- 2 minutes: Pair with a neighbor to discuss. * 2 minutes: Briefly share ideas with the class (leads into stress concentration and testing).

## III. Design Categories & Decisions (20 Minutes)

- The Four Categories: Categorize projects based on scale and risk:

  - High Risk/High Volume: Endangers life; justifies elaborate testing.

  - Moderate Volume: Moderate testing is feasible.

  - Low Volume/Rapid Prototype: No time/justification for testing.

  - Post-Failure Analysis: Redesigning an unsatisfactory part.

### Active Learning: Small Group Discussion

Prompt: You are hired at a startup with limited resources and no testing lab, but your product requires high accuracy. What measures do you take?

- Group Task (10 mins): Assign students into groups representing the four design categories. Have them list one "safety measure" unique to their category.

## IV. Static Loading Fundamentals (15 Minutes)

- Static Load Definition: A stationary force/couple that does not change in magnitude, direction, or point of application.

- Load Types: These produce axial tension/compression, shear, bending, torsion, or combinations.

### Active Learning: Identification Challenge

Prompt: Show 3 images of mechanical components (e.g., a bolt, a driveshaft, a bracket). Have students identify the load type (Axial, Shear, Torsion) and determine if it meets the "Static" criteria.

## V. Failure Theories: Ductile vs. Brittle (15 Minutes)

- Identification:

  - Ductile: Failure strain $\varepsilon_f \geq 0.05$. Focuses on yield strength.

  - Brittle: Failure strain $\varepsilon_f < 0.05$. No identifiable yield; focuses on ultimate stress.

- The Theories: There is no universal theory, only "accepted practices".

- Ductile Criteria: MSS, DE, and DCM.

- Brittle Criteria: MNS, BCM, and MM.

## VI. Summary & Wrap-up (5 Minutes)

- Recap: We design based on material behavior (ductile vs. brittle) and the statistical uncertainty of strength.

- Exit Ticket: Have students write down which failure theory (e.g., MSS or DE) they think is most conservative for ductile materials based on their reading.

Prompt (fast version):

I am an assistant professor teaching Elements of mechanical design course to third year mechanical engineering BS students. I want to create a 75 minute long lecture using the slide deck attached focusing on failure and failure theories. Please note 2-3 learning objectives and some active learning exercises to achieve the said learning objectives. Students will be working in pairs.

This lecture plan for MEEG 3379: Elements of Mechanical Design is designed to transform static slide content into an engaging 75-minute session. By focusing on the statistical nature of strength and the logic behind failure theories, students will move beyond rote memorization toward engineering intuition.

## Lecture Learning Objectives

By the end of this 75-minute session, students will be able to:

- Differentiate between the four design categories to determine the appropriate level of testing and analysis required for a given project.

- Evaluate material behavior (ductile vs. brittle) using failure strain ($\varepsilon_f$) to select the correct class of failure theories.

- Justify why published material strengths cannot be used as absolute values in mechanical design due to statistical variability.

## 75-Minute Lecture Timeline

### 1. Introduction & Logistics (5 Minutes)

- Instructor Intro: Dr. Shweta Dabetwar (Office: EIT 544).

- Support: Introduce TA Colby Martinez.

- Materials: Reference Shigley’s Mechanical Engineering Design, 11th Ed.

### 2. Redefining Failure (10 Minutes)

- Core Concept: Failure occurs when a component can no longer perform its intended job.

- The Four Faces of Failure: Separation, permanent distortion, downgraded reliability, and compromised function.

- The Root Cause: Diminished strength.

### 3. The Statistical Nature of Design (15 Minutes)

- The Testing Paradox: Even with extensive testing, parts fail because mass-produced items vary in dimension, machining, and composition.

- Statistical Strength: Strength is a distribution (mean and standard deviation), not a single number; therefore, published values are insufficient on their own.

- Operational Risk: Stress concentrations during actual operation can deviate significantly from ideal test conditions.

### 4. Active Learning I: The "Design Category" Dilemma (15 Minutes)

- Strategy: Think-Pair-Share (using Slide 4 and 5 content).

- The Setup: Introduce the four design categories ranging from life-safety/high-volume to post-failure analysis.

- The Prompt: "You are hired at a startup with limited resources and no testing lab, but your product requires high accuracy. Which design category does this fall into, and what measures do you take?".

- Activity:

  - Think (2 min): Students write their individual strategies.

  - Pair (5 min): Partners compare strategies and select the safest, most cost-effective path.

  - Share (8 min): The professor calls on pairs to justify their "measures" (e.g., higher factors of safety, conservative modeling).

### 5. Static Loading & Material Behavior (15 Minutes)

- Defining Static Load: A load stationary in magnitude, direction, and point of application.

- Resulting Stresses: Axial, shear, bending, torsion, or combinations.

- The Ductile/Brittle Divide: * Ductile: $\varepsilon_f \geq 0.05$; fails via yielding; tensile/compressive strengths are similar.

  - Brittle: $\varepsilon_f < 0.05$; fails via fracture; tensile/compressive strengths differ.

### 6. Active Learning II: Theory Selection Matrix (10 Minutes)

- Strategy: Peer Instruction / Categorization.

- Activity: Provide a list of hypothetical materials and their failure strains. In pairs, students must map these materials to the "Generally Accepted Failure Theories".

- Task: * If $\varepsilon_f = 0.12$, which theories apply? (Expected: MSS, DE, DCM) .

  - If $\varepsilon_f = 0.01$, which theories apply? (Expected: MNS, BCM, MM) .

### 7. Conclusion & "Muddiest Point" (5 Minutes)

- Summary: There is no universal theory; we rely on "accepted practices" developed over years of hypotheses.

- Closing Activity: Each student writes down the "muddiest point" (the concept they found most confusing) on a post-it or digital form to guide the start of the next lecture.

Same prompt (Pro version):

Here is a comprehensive 75-minute lecture plan designed for your third-year MEEG 3379 students. It breaks down your slide deck into digestible segments and integrates pair-based active learning exercises to transition them from passive listening to active application.

### Learning Objectives

By the end of this 75-minute lecture, students will be able to:

- Define mechanical failure beyond physical breakage and explain why statistical variations prevent the direct use of published material strengths.

- Evaluate a real-world engineering scenario to determine the appropriate design and testing category.

- Classify material behavior as ductile or brittle based on failure strain and select the corresponding class of failure theories.

### Lecture Timeline & Plan (75 Minutes Total)

#### Part 1: Introduction & Logistics (5 Minutes)

- Welcome & Course Logistics: Introduce yourself, your office hours (Thursdays, 12:15-1:15 pm, EIT 544) , and the TA, Colby Martinez.

- Material: Confirm the textbook is Shigley's Mechanical Engineering Design, 11th Edition, Chapter 5.

#### Part 2: Redefining Failure & The Statistical Reality (15 Minutes)

- What is Failure? Define failure as when a system or component can no longer perform its intended job. Emphasize that failure is not just breaking; it includes permanent distortion, downgraded reliability, and compromised function. The root cause is diminished strength.

- The Testing Paradox: Pose the question: If parts undergo extensive testing, why do they still fail?.

- Statistical Strength: Explain that mass-produced parts have different strengths due to variations in dimensions, machining, forming, and composition. Explain that strength is statistical in nature, meaning we cannot rely solely on published values for yield or ultimate strength.

Active Learning Exercise 1: The "Book Value" Dilemma (5 Minutes)

- Format: Think-Pair-Share.

- Prompt: Ask students to pair up. "Since strength is a statistical distribution and stress concentrations during operation can throw everything off, how should we account for this uncertainty mathematically when designing a critical component?"

- Action: Pairs discuss for 3 minutes. Call on 2-3 pairs to share. (Goal: Guide them toward the concept of safety factors and conservative design).

#### Part 3: The Four Design Categories (20 Minutes)

- Categorizing Design: Explain that testing must duplicate actual design situations. Introduce the four categories based on risk and volume:

  - High Risk / Huge Volume: Failure endangers human life or involves extreme quantities; elaborate testing is justified.

  - Moderate Volume: Large enough quantities to justify a moderate series of tests.

  - Low Volume / Rapid Timeline: Testing is not justified, or the design must be completed too rapidly.

  - Post-Failure Analysis: Redesigning a manufactured part that was found to be unsatisfactory.

Active Learning Exercise 2: Scenario Analysis (10 Minutes) * Format: Pair Discussion & Strategy Formulation. * Prompt: "You are hired at a startup with limited resources and less testing facilities, but your product needs very high accuracy. What measures would you take?"

- Action: Give pairs 5 minutes to develop a 3-step action plan for this specific scenario (Category 3). Have a few pairs present their strategies (e.g., over-engineering, using well-documented materials, focusing resources on computer simulation).

#### Part 4: Static Loading & Material Behavior (15 Minutes)

- Defining Static Load: Define it as a stationary force or couple where magnitude, direction, and points of application do not change.

- Load Types: List axial, shear, bending, torsional, or combinations.

- Ductile vs. Brittle Identification:

  - Ductile: Failure strain $\varepsilon_f \geq 0.05$. There is an identifiable yield strength. The maximum stress is the yield stress, and tensile/compressive values are similar.

  - Brittle: Failure strain $\varepsilon_f < 0.05$. There is no identifiable yield strength. The maximum stress is ultimate stress, and tensile/compressive values usually differ.

#### Part 5: Failure Theories Map (15 Minutes)

- No Universal Theory: Explain that there is no single universal theory; we rely on hypotheses that have become "accepted practices".

- The Breakdown: * Yield Criteria (Ductile): Maximum shear stress (MSS), Distortion energy (DE), and Ductile Coulomb-Mohr (DCM).

  - Fracture Criteria (Brittle): Maximum normal stress (MNS), Brittle Coulomb-Mohr (BCM), and Modified Mohr (MM).

Active Learning Exercise 3: The Theory Selector (10 Minutes)

- Format: Pair Problem Solving.

- Prompt: Project two hypothetical materials on the board: Material A ($\varepsilon_f = 0.02$) and Material B ($\varepsilon_f = 0.15$).

- Action: Pairs must identify which material is ductile and which is brittle, state the governing maximum stress parameter (yield vs. ultimate), and list the valid failure theories for each. This solidifies their ability to connect strain data directly to design criteria.

#### Part 6: Summary & Wrap-Up (5 Minutes)

- Recap: Briefly summarize the transition from defining failure to selecting the proper failure theory.

- Look Ahead: Inform them that the next lecture will dive deep into the math behind the Ductile Yield Criteria (MSS and DE). Remind them of Thursday's office hours.